NAME : Pushti Shah                                                                            SAP ID: 60019210097

ROLL NO: B074                                                                                  BRANCH: CSE-ICB

# Experiment 2

# Aim: Implementation of VLAN (Layer‑2 Security)

## Objective:

In this lab, students will:

- Install Cisco Packet Tracer 9.0.0

- Learn how to navigate the Packet Tracer interface

- Create a simple network topology

- Configure VLANs on a switch

- Assign ports to VLANs

- Verify VLAN isolation (Layer‑2 security)

- Test connectivity using simulation tools

This document contains step‑by‑step instructions with mouse clicks, icons, colors, and UI navigation details so beginners can follow easily.

# Part 1 – Download and Install Cisco Packet Tracer 9.0.0

### Step 1 – Open the Download Link

1. Open Google Chrome / Edge / Firefox.

2. In the address bar (top white box), paste this link:https://archive.org/download/cisco-packet-tracer-900-win-64bit/CiscoPacketTracer_900_win_64bit.exe

3. Press Enter key.

### Step 2 – Download the Installer

1. The file will start downloading automatically.

2. You will see the download progress at the bottom of the browser.

3. Wait until the file is fully downloaded.

File name should look like:

CiscoPacketTracer_900_win_64bit.exe

### Step 3 – Run the Installer

1. Go to your Downloads folder.

2. Double‑click on:CiscoPacketTracer_900_win_64bit.exe

3. If Windows shows a security popup:

  - Click Yes button.

### Step 4 – Installation Wizard

1. The setup window opens.

2. Click Next button (blue button).

3. Accept the license agreement:

  - Select I Agree radio button.

  - Click Next.

4. Choose install location (keep default).

5. Click Next.

6. Click Install.

7. Wait for installation to finish.

8. Click Finish.

A desktop shortcut icon will appear:

Cisco Packet Tracer icon

# Part 2 – Launching Cisco Packet Tracer

1. Double‑click the Cisco Packet Tracer desktop icon.

2. If login screen appears:

  - Click Login.

  - It will redirect you to login page on browser.

Main application window will open.

# Part 3 – Understanding Packet Tracer Interface

### Main Sections of Screen

#### 1. Menu Bar (Top)

- File

- Edit

- Options

- View

- Help

Used for saving, opening, preferences, etc.

#### 2. Toolbar (Below Menu)

Icons include:

- 📁 New file

- 💾 Save

- 🔍 Zoom In / Out

- ▶️ Simulation controls

#### 3. Workspace (Center White Area)

- This is where you build the network.

- You drag and drop devices here.

#### 4. Device Selection Panel (Bottom Left)

Contains icons:

🖥️ End Devices

- 🔀 Switches

- 🌐 Routers

- ⚡ Connections

#### 5. Device Models Panel (Bottom Right)

Shows specific models when you select a category.

Example:

- Switch → 2960

- PC → PC‑PT

# Part 4 – Creating Network Topology

## 🖧 Network Design

We will create:

- 1 Switch

- 4 PCs

PC1, PC2 → VLAN 10
PC3, PC4 → VLAN 20

### Step 1 – Add a Switch

1. In bottom left panel, click Switches icon (🔀).

2. In bottom right panel, click 2960 switch.

3. Click once on the white workspace to place the switch.

### Step 2 – Add PCs

1. Click End Devices icon (🖥️).

2. Click PC‑PT.

3. Click on workspace four times to place 4 PCs.

Rename PCs:

1. Click PC → Config tab → Display Name

2. Rename as:

  - PC1

  - PC2

  - PC3

  - PC4

### Step 3 – Connect Devices

1. Click Connections icon (⚡ yellow lightning icon).

2. Select Copper Straight‑Through cable (black line icon).

Connect:

- PC1 → Switch FastEthernet0/1

- PC2 → Switch FastEthernet0/2

- PC3 → Switch FastEthernet0/3

- PC4 → Switch FastEthernet0/4

To connect:

- Click PC → click Switch port

Green dots indicate successful connection.

# Part 5 – Assign IP Addresses

### For Each PC

1. Click PC.

2. Click Desktop tab.

3. Click IP Configuration.

4. Enter values:

PC1:

IP: 192.168.10.10    
    Subnet: 255.255.255.0

PC2:

IP: 192.168.10.11    
    Subnet: 255.255.255.0

PC3:

IP: 192.168.20.10    
    Subnet: 255.255.255.0

PC4:

IP: 192.168.20.11    
    Subnet: 255.255.255.0

Close window.

# Part 6 – Configure VLANs on Switch

### Open Switch CLI

1. Click on Switch.

2. Click CLI tab.

3. Press Enter until you see:

Switch>

### Create VLANs

Type commands exactly:

enable    
    configure terminal    
    vlan 10    
    name SALES    
    exit    
    vlan 20    
    name HR    
    exit

### Assign Ports to VLANs

interface fastEthernet0/1    
    switchport mode access    
    switchport access vlan 10    
    exit    
         
    interface fastEthernet0/2    
    switchport mode access    
    switchport access vlan 10    
    exit    
         
    interface fastEthernet0/3    
    switchport mode access    
    switchport access vlan 20    
    exit    
         
    interface fastEthernet0/4    
    switchport mode access    
    switchport access vlan 20    
    exit    
    exit

### Verify VLAN Configuration

show vlan brief

You should see ports mapped to VLANs.

Switch#show vlan brief    
         
    VLAN Name                             Status    Ports    
    ---- -------------------------------- --------- -------------------------------    
    1    default                          active    Fa0/5, Fa0/6, Fa0/7, Fa0/8    
                                                    Fa0/9, Fa0/10, Fa0/11, Fa0/12    
                                                    Fa0/13, Fa0/14, Fa0/15, Fa0/16    
                                                    Fa0/17, Fa0/18, Fa0/19, Fa0/20    
                                                    Fa0/21, Fa0/22, Fa0/23, Fa0/24    
                                                    Gig0/1, Gig0/2    
    10   SALES                            active    Fa0/1, Fa0/2    
    20   HR                               active    Fa0/3, Fa0/4    
    1002 fddi-default                     active    
    1003 token-ring-default               active    
    1004 fddinet-default                  active    
    1005 trnet-default                    active

# Part 7 – Testing VLAN Security

### Ping Test

1. Click PC1.

2. Desktop → Command Prompt.

3. Type:

ping 192.168.10.11

Ping should succeed.

### Cross VLAN Test

From PC1:

ping 192.168.20.10

Ping should fail.

This proves VLAN isolation.

# Part 8 – Simulation Mode (Optional)

1. Click Simulation button (bottom right).

2. Click Play button ▶️.

3. Watch packet flow animation.

OUTPUT:

Conclusion:

In this experiment, we successfully implemented VLAN (Virtual Local Area Network) to achieve Layer-2 security using Cisco Packet Tracer. We learned how to install and navigate the tool, create a network topology, and configure VLANs on a switch.

By assigning different ports to separate VLANs (VLAN 10 and VLAN 20), we logically divided the network into isolated segments. Devices within the same VLAN were able to communicate successfully, while communication between different VLANs failed, demonstrating proper network isolation and enhanced security.

This experiment helped in understanding how VLANs improve network management, reduce broadcast domains, and provide security by restricting unauthorized access between departments. Overall, VLAN configuration is an essential concept in modern networking for building secure and efficient networks.